Open your environment : conda activate ddos-py311
cd ~/DDoS-Detection-SDN/src

LATEST BACKUP DONE OF WORKING RYU: cp ryu_ddos_controller.py ryu_ddos_controller_$(date +%Y%m%d_%H%M)_attackH_OK.py      (H is your backup can be any code)

VERIFY IT EXISTS

ls -lh ryu_ddos_controller_*attackC_OK.py.  or    cp ryu_ddos_controller.py ryu_ddos_controller_FINAL.py

# Attack B (TCP SYN Flood) – Experimental Evidence

## 1. Attack Generation (Mininet)

A TCP SYN flood attack was launched from host h1 targeting host h4 on port 8000 using hping3.

### Command

mininet> h1 hping3 -S --flood -p 8000 10.0.0.4

### Output

HPING 10.0.0.4 (h1-eth0 10.0.0.4): S set, 40 headers + 0 data bytes

hping in flood mode, no replies will be shown

^C

--- 10.0.0.4 hping statistic ---

18218935 packets transmitted, 0 packets received, 100% packet loss

round-trip min/avg/max = 0.0/0.0/0.0 ms

### Ryu Controller Output

Blocked flow: {'ipv4_src': '10.0.0.4', 'ipv4_dst': '10.0.0.1',

'ip_proto': 6, 'tcp_src': 8000, 'tcp_dst': 1537} prob 4712.26513671875

FastAPI response:

{'prediction': 'DDoS Attack',

'probability': 4712.85595703125,

'raw_output': [-5556.30126953125, 4712.85595703125]}

PktIn: 10.0.0.4 -> 10.0.0.1 proto 6 ports 8000 -> 1538

prediction DDoS Attack prob 4712.85595703125

Blocked flow:

{'ipv4_src': '10.0.0.4', 'ipv4_dst': '10.0.0.1',

'ip_proto': 6, 'tcp_src': 8000, 'tcp_dst': 1538}

prob 4712.85595703125

### Switch Flow Table (s2)

mininet> sh ovs-ofctl dump-flows s2 | grep drop

priority=200,tcp,tp_dst=8000 actions=drop

The FastAPI inference server exported attack metrics.

### Metrics Endpoint

curl -s http://127.0.0.1:8000/metrics | grep ddos_inference_total

### Output

# HELP ddos_inference_total Total inference requests

# TYPE ddos_inference_total counter

ddos_inference_total{result="DDoS Attack"} 56.0

### Prometheus Query

curl -s "http://127.0.0.1:9090/api/v1/query?query=ddos_inference_total" | jq

### Output

{

"metric": {

"__name__": "ddos_inference_total",

"instance": "127.0.0.1:8000",

"job": "ddos_fastapi",

"result": "DDoS Attack"

},

"value": [

1771156190.360,

"56"

]

}

Grafana was configured with Prometheus as a data source:

http://127.0.0.1:9090

Attack C simulates a high-rate ICMP Echo Request flood aimed at:

- Saturating link bandwidth

- Overloading switch forwarding tables

- Increasing controller packet-in load

- Degrading legitimate traffic

Unlike SYN floods, this attack targets network capacity and forwarding paths.

mininet> h1 hping3 --icmp --flood 10.0.0.4

During Attack C, Ryu should print:

PktIn: 10.0.0.1 -> 10.0.0.4 proto 1

FastAPI response: {...}

prediction: DDoS Attack

Blocked flow: {...}

In your Ryu controller, confirm you have:

if ip_pkt and ip_pkt.proto == 1:   # ICMP

if ml_prob > SUSPICIOUS_THRESHOLD:

match = parser.OFPMatch(

eth_type=0x0800,

ip_proto=1,

ipv4_src=src_ip,

ipv4_dst=dst_ip

)

block_flow(datapath, match)

print("ICMP flood blocked")

If this is not present, Attack C will not be mitigated.

After running attack:

mininet> sh ovs-ofctl dump-flows s2 | grep icmp

You should see:

priority=200,ip_proto=1 actions=drop

## Prometheus Verification

Check:

curl -s http://127.0.0.1:9090/api/v1/query?query=ddos_inference_total | jq

Expect:

result="DDoS Attack"

## to increase during attack.  Grafana Visualization

Panel query for Attack C:

rate(ddos_inference_total{result="DDoS Attack"}[1m])

You should observe:

- Spike during ping flood

- Flat baseline after mitigation

- Save:

- (a) Attack

- h1 hping3 --icmp --flood 10.0.0.4

- (b) Ryu Logs

- FastAPI response

- ICMP flood blocked

- Blocked flow

- (c) OVS Rules

- ovs-ofctl dump-flows s2

- (d) Grafana Screenshot

- During spike.

#### Attack D : Start a DNS responder on the reflector (h2)

On h2:

mininet> h2 python3 -m http.server 53 &

#### (real UDP responder) on h2

mininet> h2 python3 - <<'PY' &

import socket

s=socket.socket(socket.AF_INET, socket.SOCK_DGRAM)

s.bind(("0.0.0.0", 53))

while True:

data, addr = s.recvfrom(4096)

# send back a bigger payload (amplification-like)

s.sendto(data + b"A"*1200, addr)

PY

#### Launch the reflection flood from attacker (h1)

Spoof victim as source, send UDP to reflector so reflector blasts victim:

mininet> h1 hping3 --udp -p 53 --flood --spoof 10.0.0.4 10.0.0.2

What you should see:

- Victim h4 becomes overwhelmed (packet loss / service degradation).

- Controller logs show high PacketIn/inference activity (many UDP events).

- Prometheus ddos_inference_total{result="DDoS Attack"} spikes.

Switch flow tables gain UDP drop rules (ip_proto=17)   Detection logic (minimal):

- Treat proto == 17 + high packet rate / repeated PacketIn as suspicious.

- Let the FastAPI DNN make the final call (keeps “learning from other attacks” consistent).

Do not hard-block all UDP. Instead, only raise suspicion when UDP port 53 is involved (DNS) or when a per-source PPS threshold is exceeded.

What to implement (conceptually):

- If proto == 17 and (src_port==53 or dst_port==53): mark suspicious.

- Optional: maintain a short sliding counter per src_ip (or per switch) for UDP PacketIn frequency.

This makes DNS reflection stand out without breaking normal UDP.  When DNN returns “DDoS Attack” and proto == 17, your existing block section should populate:

- ip_proto=17

- udp_src / udp_dst when available

### hot-reload already exists

You already have app_pytorch_dynamic.py with watcher reloads.
So your “learning loop” is:

- run attacks → collect labeled inference logs

- retrain offline → output new dnn_model.pt

- watcher reloads model → metrics show ddos_model_reloads_total increment

- re-run attacks → compare detection/latency

recommending the port-53 suspicion trigger: it boosts separability without rewriting your model.

### Port 53 binding and Mininet privileges

- Binding to port 53 requires root privileges on Linux.

- In Mininet, commands typically run as root, so it often works—but if you run Mininet differently, it may fail.

(spoofed source → reflector replies to victim). It becomes amplification only if:

- request is small, response is large (your + b"A"*1200 does that), and/or

- you send many small requests very fast.

Recommended (use high port 5353 to avoid privilege issues):

mininet> h2 python3 - <<'PY' &

import socket

s = socket.socket(socket.AF_INET, socket.SOCK_DGRAM)

s.bind(("0.0.0.0", 5353))

print("UDP amplifier listening on 5353")

while True:

data, addr = s.recvfrom(4096)

# amplification-like: respond with much larger payload

s.sendto(data + b"A"*1200, addr)

PY

If you insist on port 53, change 5353 → 53 (works only if allowed to bind).

If you used 5353 above:

mininet> h1 hping3 --udp -p 5353 --flood --spoof 10.0.0.4 10.0.0.2

If you used 53 above:

mininet> h1 hping3 --udp -p 53 --flood --spoof 10.0.0.4 10.0.0.2

Recommended gate

- If ip_proto == 17 AND (udp_src == 53/5353 OR udp_dst == 53/5353), raise a suspicion flag / lower threshold / always run DNN.

- Optional: maintain a short rolling counter (per src_ip or per switch) for UDP PacketIn rate and trigger DNN when it exceeds a threshold.

This avoids “block all UDP” while making Attack D separable without redesigning your ML features.

# Confirm switches are connected to Ryu

In Mininet run:

mininet> sh ovs-vsctl show

You must see something like:

Controller "tcp:127.0.0.1:6633"
is_connected: true

If it says is_connected: false, the switches are not connected.

# Confirm table-miss rule is actually correct

Run:

mininet> sh ovs-ofctl -O OpenFlow13 dump-flows s1

The correct rule must look like:

priority=0 actions=CONTROLLER:65535

If it says something else, the controller will not receive packets.

# Force a PacketIn test

Run:

mininet> h1 ping -c 1 h2

Watch the Ryu terminal.

You should see something like:

packet_in_handler called

If nothing prints, your packet_in_handler decorator is probably not attached correctly.

It must look exactly like this:

@set_ev_cls(ofp_event.EventOFPPacketIn, MAIN_DISPATCHER)
def packet_in_handler(self, ev):

and must not be outside the class indentation.

# If packet_in_handler never fires

Check indentation of this section:

class SimpleDDOSController(app_manager.RyuApp):

The handler must be inside this class, not at column 0.

Correct example:

class SimpleDDOSController(app_manager.RyuApp):

    @set_ev_cls(ofp_event.EventOFPPacketIn, MAIN_DISPATCHER)
    def packet_in_handler(self, ev):
        ...

# If it is outside the class, Ryu will never call it. Why tcpdump showed nothing

You ran:

h2 tcpdump -n udp port 5353

But the switch already installed:

actions=drop

So packets never reach the host, which is correct behaviour.

Since everything works:

cp ryu_ddos_controller.py ryu_ddos_controller_ATTACKD_VERIFIED_$(date +%Y%m%d_%H%M).py

# Attack E.    Start an NTP-like reflector in Mininet

Run on h2:

mininet> h2 python3 - <<'PY' &
import socket
s = socket.socket(socket.AF_INET, socket.SOCK_DGRAM)
s.bind(("0.0.0.0",123))
print("NTP amplifier running on port 123")

while True:
    data, addr = s.recvfrom(4096)
    s.sendto(data + b"B"*2000, addr)
PY

You should see:

NTP amplifier running on port 123

From h1:

mininet> h1 hping3 --udp -p 123 --flood --spoof 10.0.0.4 10.0.0.2

This simulates:

bot → spoofed victim → NTP reflector → amplified response

While the attack runs:

mininet> sh ovs-ofctl -O OpenFlow13 dump-flows s2 | grep udp

Expected:

ip_proto=17
udp_dst=123

Packet counters should increase rapidly.

# Expected mitigation rule

mininet> sh ovs-ofctl -O OpenFlow13 dump-flows s2 | grep drop

Example:

priority=200,udp,nw_src=10.0.0.4,nw_dst=10.0.0.2 actions=drop

# Attack F.    Start a target service

Run a simple TCP service on h2:

mininet> h2 python3 -m http.server 80 &

# This creates a listening TCP service.  Launch the SYN flood

From h1 run:

mininet> h1 hping3 -S -p 80 --flood --rand-source 10.0.0.2

Explanation:

| Option | Meaning |
| --- | --- |
| -S | SYN packets |
| -p 80 | target port |
| --flood | high-rate attack |
| --rand-source | spoofed addresses |

# Verify attack traffic

Run during attack:

mininet> sh ovs-ofctl -O OpenFlow13 dump-flows s2

# You should see packet counters increasing rapidly. Expected controller behaviour

Your controller should log something like:

PktIn: proto 6 ports ...
prediction DDoS Attack
Blocked flow

TCP protocol number:

# proto = 6.  Mitigation rule example

Your controller will install something similar to:

priority=200,tcp,nw_dst=10.0.0.2,tp_dst=80 actions=drop

This blocks the SYN flood in the switch.

# Only add this if you want a fast explicit SYN rule (optional)

You can add a SYN flood heuristic before the ML block to reduce controller load.

Insert after:

ip_pkt = pkt.get_protocol(ipv4.ipv4)

### Optional SYN detection

# -------- ATTACK F: TCP SYN FLOOD -------
tcp_pkt = pkt.get_protocol(tcp.tcp)

if ip_pkt and ip_pkt.proto == 6 and tcp_pkt:
    if tcp_pkt.bits & tcp.TCP_SYN and not (tcp_pkt.bits & tcp.TCP_ACK):

        match = parser.OFPMatch(
            eth_type=0x0800,
            ip_proto=6,
            ipv4_src=ip_pkt.src,
            ipv4_dst=ip_pkt.dst,
            tcp_dst=tcp_pkt.dst_port
        )

        block_flow(datapath, match)
        print("TCP SYN flood blocked")
        return
# ----------------------------------------

# This simply detects SYN without ACK.  If you do NOT add this

Your system still works:

SYN packets → PacketIn → ML model → detection → block_flow

But:

- controller load will be higher

- detection will occur after ML inference

- Correct command to run Attack F

- mininet> h1 hping3 -S --flood -p 80 10.0.0.2

- or with spoofing:

- mininet> h1 hping3 -S --flood --rand-source -p 80 10.0.0.2

- What you should see

- Controller:

- PktIn proto 6
prediction DDoS Attack
Blocked flow

- Switch:

- ovs-ofctl dump-flows s2 | grep drop
priority=200 tcp,nw_src=...,nw_dst=... actions=drop

# Attack G.   Remove forwarding rules

Restart Mininet so switches have only table-miss rules.

sudo mn -c

# Then restart your topology.  Launch random-flow attack

From h1:

mininet> h1 hping3 --flood --rand-source --rand-dest 10.0.0.2

or more controlled:

mininet> h1 hping3 --flood --rand-source -p ++1 10.0.0.2

This creates many unique flows.

Each new flow causes:

# Switch → PacketIn → Controller.   Observe controller load

Your Ryu logs will show many events:

DEBUG: packet_in_handler called
PktIn: ...

# This indicates control-plane stress.  Measure PacketIn rate (important metric)

Run:

watch -n 1 "ovs-ofctl dump-flows -O OpenFlow13 s2"

Look at:

n_packets

Also observe Prometheus metrics:

ddos_inference_total

# These should spike during the attack. Expected mitigation behaviour

Once ML detects abnormal traffic patterns:

prediction DDoS Attack
Blocked flow

Controller installs:

priority=200 actions=drop

## This reduces PacketIn events.   Good mixed scenario

Use:

- Normal traffic: iperf3 or simple HTTP between legitimate hosts

- Attack traffic: Attack D, E, or F from the attacker host

## Example setup in Mininet

### Normal flow

On h4 start a server:

mininet> h4 iperf3 -s &

On h3 start a normal client flow:

mininet> h3 iperf3 -c 10.0.0.4 -t 60 &

That gives you a steady benign flow.

### While that is running, launch the attack

For example Attack D:

mininet> h1 hping3 --udp -p 5353 --flood --spoof 10.0.0.4 10.0.0.2

Attack H.